## Gas Distribution in Unventilated Indoor Environments Inspected by a Mobile Robot (2003)

Wandel, Lilienthal, Duckett, Weimar & Zell. ICAR.

- First systematic robot survey of how gas actually distributes in a real unventilated room.

- Showed the distribution is patchy and driven by turbulence and convection, not smooth diffusion.

- Established the empirical problem that the rest of Lilienthal's program sets out to formalize.

## Building Gas Concentration Gridmaps with a Mobile Robot (2004)

Lilienthal & Duckett. Robotics and Autonomous Systems 48(1). Precursor: IROS 2003.

- The founding gas distribution mapping (GDM) paper.

- Applies a Gaussian kernel to spread each reading over nearby cells, modeling the decreasing likelihood that a measurement represents true concentration further from the sample point.

- Confronts the core problem that a gas reading informs only a tiny area, unlike sonar or laser.

- Proposes using the resulting map to locate the source, which is the seed of the mapping-first approach to OSL.

## Experimental Analysis of Gas-Sensitive Braitenberg Vehicles (2004)

Lilienthal & Duckett. Advanced Robotics 18(8).

- Tests purely reactive tropotaxis using an instantaneous spatial gradient from a bilateral stereo nose.

- Finds both positive and negative strategies have some use indoors, but exposes their brittleness in turbulence.

- Important as the bio-inspired baseline Lilienthal later argues against.

## Gas Source Declaration with a Mobile Robot (2004)

Lilienthal, Ulmer, Fröhlich, Stützle, Werner & Zell. ICRA.

- Isolates gas source declaration as a distinct subtask: deciding whether a candidate object is actually the source.

- Uses SVM classification on readings taken during a rotation maneuver in front of the candidate.

- Validated over 1056 declaration trials at varying robot-to-source distances.

- Early proof that statistical and machine learning methods beat hand-tuned heuristics on OSL subtasks.

## Gas Distribution Mapping and Gas Source Localisation with a Mobile Robot (PhD thesis) (~2004)

Lilienthal. University of Tübingen, later published by Shaker Verlag.

- Consolidates the gridmap, Braitenberg and source-declaration threads into one framework.

- States the central thesis: in turbulent real environments, GDM plus statistical inference is the productive route to OSL.

## Indicators of Gas Source Proximity using Metal Oxide Sensors in a Turbulent Environment (2006)

Lilienthal, Duckett, Werner & Ishida. BioRob.

- Looks for signal features that reliably correlate with proximity to the source under turbulence.

- Bridges raw MOX sensing and the declaration/localization decision.

- Notable as a link between the Örebro and Ishida (Tokyo) research lines.

## Airborne Chemical Sensing with Mobile Robots (2006)

Lilienthal, Loutfi & Duckett. Sensors 6(11).

- The field-defining survey of the era, still the standard orientation paper.

- Formalizes the taxonomy the community still uses: gas distribution mapping, trail guidance, and gas source localization (split into tracing, declaration, localization).

- Deliberately restricts itself to experimental work, arguing that gas distribution cannot yet be realistically simulated, which is itself a methodological stance.

## A Rao-Blackwellisation Approach to GDM-SLAM (2007)

Lilienthal, Loutfi, Blanco, Galindo & Gonzalez. ECMR.

- Couples gas distribution mapping with SLAM so pose uncertainty propagates into the gas map.

- Removes the unrealistic assumption that the robot always knows exactly where it sampled.

## Gas Distribution Modeling using Sparse Gaussian Process Mixtures (2008-09)

Stachniss, Plagemann, Lilienthal & Burgard. RSS 2008; Autonomous Robots 26(2-3).

- Recasts GDM as a regression problem solved with a sparse Gaussian process mixture.

- Separates a smooth background component from high-concentration patches, and outputs predictive uncertainty alongside the mean.

- Beat prior robotics methods on real e-nose data, but O(n³) training cost motivated the cheaper kernel methods that followed.

## A Statistical Approach to Gas Distribution Modelling with Mobile Robots: the Kernel DM+V Algorithm (2009)

Lilienthal, Reggente, Trincavelli, Blanco & Gonzalez. IROS.

- The single most influential methodological contribution in the arc, and the hinge between early and modern OSL.

- Estimates per-cell distribution mean (DM) and predictive variance (V) via two parallel kernel extrapolation processes.

- The variance map matters directly for OSL: high-fluctuation regions correlate with source proximity.

- Far cheaper than the GP mixture, making statistical GDM practical to deploy on real robots.

## Gas Distribution Mapping of Multiple Odour Sources using a Mobile Robot (2009)

Loutfi, Coradeschi, Lilienthal & Gonzalez. Robotica.

- Extends kernel gridmapping to multiple, chemically distinct sources.

- Couples gas discrimination with mapping rather than assuming a single known analyte.

## Using Local Wind Information for Gas Distribution Mapping (Kernel DM+V/W) and the 3D-Kernel DM+V/W Algorithm (2009-10)

Reggente & Lilienthal. IEEE Sensors.

- Deforms the kernel according to measured airflow direction and magnitude.

- Large accuracy gains outdoors and under strong wind, where isotropic kernels fail badly.

- Extends the model from 2D floor plans to 3D volumes.

## Gas Discrimination for Mobile Robots and Odour Classification for Continuous Monitoring (2009-11)

Trincavelli et al. Sensors and Actuators B; KI Künstliche Intelligenz.

- Establishes that in open sampling, MOX sensors almost never reach steady state because of turbulence.

- Classification must therefore use the transient portion of the response, via curve fitting and Fourier or wavelet features with an SVM.

- Tested across a closed room, indoor corridors and an outdoor courtyard.

- Sensor dynamics as a first-class concern becomes a recurring Lilienthal theme.

## TD Kernel DM+V: Time-Dependent Statistical Gas Distribution Modelling (2011)

Asadi, Pashami, Loutfi & Lilienthal. ISOEN 2011, extended in Robotics and Autonomous Systems 96 (2017).

- Adds a recency weight so older measurements decay in influence.

- Drops the false assumption that the gas distribution is a stationary process.

- Makes GDM usable for monitoring evolving or intermittent releases.

## Mobile Robots for Localizing Gas Emission Sources on Landfill Sites: Is Bio-Inspiration the Way to Go? (2011/12)

Hernandez Bennetts, Lilienthal, Neumann & Trincavelli. Frontiers in Neuroengineering 4:20.

- The manifesto of the anti-biomimetic position and the clearest statement of the field's reframing.

- Argues biomimetic OSL has limited value because animals differ fundamentally from robots in both sensing and actuation.

- Proposes engineered, statistics-based GSL as the alternative, with landfill methane monitoring as the reference application.

## Autonomous Gas-Sensitive Microdrone: Wind Vector Estimation and Gas Distribution Mapping (2012)

Neumann, Asadi, Lilienthal, Bartholmai & Schiller. IEEE Robotics & Automation Magazine 19(1).

- Brings GDM into the air on a microdrone.

- Estimates the wind vector in real time from the drone's own control signals, with no separate anemometer.

- Uses an artificial-potential-field adaptive sampling strategy, validated over 5x20 m outdoor areas.

- Companion 2013 paper in Advanced Robotics 27(9) directly compares bio-inspired against particle-filter OSL on the same platform.

## Towards Real-World Gas Distribution Mapping and Leak Localization Using a Mobile Robot with 3D and Remote Gas Sensing Capabilities (Gasbot) (2013)

Hernandez Bennetts, Lilienthal, Khaliq, Pomareda Sese & Trincavelli. ICRA.

- Introduces remote, open-path (TDLAS) measurement into robotic GDM.

- Platform combines LIDAR, a pan-tilt unit and a remote methane leak detector to build 3D methane maps from integral concentration plus depth.

- Tested in an underground corridor and a decommissioned landfill, so a real deployment rather than a lab mock-up.

## A Kalman Filter Based Approach to Probabilistic Gas Distribution Mapping (2013)

Blanco, Monroy, Lilienthal & Gonzalez-Jimenez. ACM SAC 2013, extended in Autonomous Robots (2016).

- Reformulates GDM as sequential Bayesian estimation on a lattice of hidden cells modeled as a Gaussian Markov random field.

- Sparse Kalman formulation gives constant-time updates instead of quadratic.

- The 2016 extension folds in obstacle and wall effects and measurement aging.

## Robot Assisted Gas Tomography: Localizing Methane Leaks in Outdoor Environments (2014)

Hernandez Bennetts, Schaffernicht, Stoyanov, Lilienthal & Trincavelli. ICRA.

- Imports computed-tomography principles into mobile robot olfaction.

- Reconstructs a spatial distribution from many integral beam measurements taken at different angles.

- Framed as convex optimization modeling both mean and fluctuation, a decisive break from point sampling.

## Using Chemical Sensors as Noses for Mobile Robots (book chapter) (2016)

Ishida, Lilienthal, Matsukura, Hernandez Bennetts & Schaffernicht. In Essentials of Machine Olfaction and Taste, Wiley.

- Consolidated reference treatment of robot olfaction sensing.

- Co-authored across the two dominant schools, Ishida's Tokyo group and Lilienthal's.

## GADEN: A 3D Gas Dispersion Simulator for Mobile Robot Olfaction in Realistic Environments (2017)

Monroy, Hernandez Bennetts, Fan, Lilienthal & Gonzalez-Jimenez. Sensors 17(7).

- ROS-based, CFD- and filament-based simulator for realistic 3D gas dispersion.

- Finally makes reproducible benchmarking of GDM and OSL algorithms possible.

- Directly answers the simulation gap flagged as a blocker in the 2006 survey.

## Bringing Mobile Robot Olfaction to the Next Dimension, and Aerial-based Gas Tomography (UAV-REGAS) (2017-19)

Neumann, Kohlhoff, Hüllmann, Lilienthal & Kluge. ICRA 2017; IEEE Sensors 2018; European Journal of Remote Sensing 52.

- Puts the tomographic remote sensor on an octocopter: gimbal-mounted TDLAS plus laser rangefinder.

- Reconstructs 2D slices through outdoor gas plumes from the air.

- Removes the ground robot's terrain and viewpoint constraints entirely.

## Smelling Nano Aerial Vehicle for Gas Source Localization and Mapping (2019)

Burgués, Hernandez, Lilienthal & Marco. Sensors 19(3).

- Then the smallest olfaction-capable flying platform: a 27 g nano-quadcopter with a custom MOX sensing board.

- Characterizes propeller-induced airflow effects on the sensor response.

- Adapts bout detection, using features from the derivative of the MOX signal, for real-time OSL.

- Targets search-and-rescue in collapsed buildings, where size is the binding constraint.

## Towards Gas Discrimination and Mapping in Emergency Response Scenarios Using a Mobile Robot with an Electronic Nose (2019)

Fan, Hernandez Bennetts, Schaffernicht & Lilienthal. Sensors 19(3).

- Pushes gas discrimination into open-world conditions where the analyte set is unknown in advance.

- Uses one-class and online learning rather than closed-set supervised classification.

- Followed by ensemble-learning extensions in Frontiers in Chemistry (2022).

## Gas Distribution Mapping and Source Localization Using a 3D Grid of MOX Sensors (2020)

Burgués, Hernandez, Lilienthal & Marco. Sensors and Actuators B 304.

- Dense volumetric sensor array instead of a single moving nose.

- Provides ground-truth-quality 3D data against which mobile-robot GDM can be evaluated.

## Sniffing Out Fugitive Methane Emissions: Autonomous Remote Gas Inspection with a Mobile Robot (ARMEx) (2021)

Arain, Hernandez Bennetts, Schaffernicht & Lilienthal. International Journal of Robotics Research 40(4-5).

- The mature ground-based remote sensing system, with an actuated spectroscopic sensor collecting integral measurements along beams up to 30 m long.

- Casts inspection as a coverage and path-planning problem that minimizes time to detection.

- Shifts the question from how to follow the plume to where to look next, making sensor planning the core of OSL.

## Sequential Deep Learning for GDM, Super-resolution for Gas Distribution Mapping, RABI-GNN and GNN-DM (2022-25)

Winkler, Kotlyar, Schaffernicht, Fan, Matsukura, Ishida, Neumann & Lilienthal. ROBOT2022; Sensors and Actuators B 419 (2024); ISOEN 2024; IEEE Sensors Journal (2025).

- The deep-learning turn in GDM, with learned priors replacing hand-designed kernels.

- Sequential models exploit spatiotemporal structure, and super-resolution reconstructs high-resolution maps from sparse samples.

- Graph neural networks handle irregular real-world sampling geometry that grid methods handle poorly.

## Exploration and Gas Source Localization in Advection-Diffusion Processes with Potential-Field-Controlled Robotic Swarms (2023)

Wiedemann, Shutin & Lilienthal et al. Sensors.

- Model-based OSL grounded in the advection-diffusion PDE rather than in kernel statistics.

- Extends from a single robot to coordinated swarms with informative sampling.

## Methane Release Rate Estimation Using Model-Based Gas Tomography, and Drone-based Mapping of Volcanic Gases (2025-26)

Schaab, Wiedemann, Hinsen & Lilienthal. IEEE Sensors Letters 9(9); arXiv.

- Couples tomography with a Lagrangian wind-advection model to estimate source release rate, not just location.

- Answers how much is leaking, which is what environmental monitoring actually needs.

- Extended to volcanic CO₂ mapping from drones.

## Deep Learning Based Topography Aware Gas Source Localization with Mobile Robot (2025)

Tian, Wang, Fan, Wiedemann, Lilienthal & Chen.

- Integrates SLAM with a U-Net that fuses gas, wind and topography to predict a source-location likelihood field.

- Terrain shapes dispersion, so ignoring it discards a strong prior, and this closes that gap.

## High-Resolution Wind Tunnel Dataset of Gas Sensor Responses to Vapor Plumes in Scale Model Landscapes (2026)

Hinsen, Wiedemann, Shutin & Lilienthal. Scientific Data.

- Public benchmark dataset with controlled ground truth.

- Addresses the field's long-standing evaluation problem, the same one the 2006 survey named twenty years earlier.